实验二实验结论

- Protues电路图

（1）8线-3线优先编码器

（2）3线-8线译码器

- 显示译码器

（4）多路选择器

- Protues仿真效果图

（1）8线-3线优先编码器

（2）3线-8线译码器

- 显示译码器

（4）多路选择器

- Quartus顶层文件原理图

（1）8线-3线优先编码器

module logic2(enb,a0,a1,a2,a3,a4,a5,a6,a7,dout0,dout1,dout2);

input enb,a0,a1,a2,a3,a4,a5,a6,a7;

output reg dout0,dout1,dout2;

always @ (enb,a0,a1,a2,a3,a4,a5,a6,a7)

if (enb==0)

begin

casex({a0,a1,a2,a3,a4,a5,a6,a7})

8'b11111111:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

end

8'b???????0:begin

dout0 <= 0;

dout1 <= 0;

dout2 <= 0;

end

8'b??????01:begin

dout0 <= 0;

dout1 <= 0;

dout2 <= 1;

end

8'b?????011:begin

dout0 <= 0;

dout1 <= 1;

dout2 <= 0;

end

8'b????0111:begin

dout0 <= 0;

dout1 <= 1;

dout2 <= 1;

end

8'b???01111:begin

dout0 <= 1;

dout1 <= 0;

dout2 <= 0;

end

8'b??011111:begin

dout0 <= 1;

dout1 <= 0;

dout2 <= 1;

end

8'b?0111111:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 0;

end

8'b01111111:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

end

default    :begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

end

endcase

end

else

begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

end

endmodule

（2）3线-8线译码器

module a74HC138(enb,a0,a1,a2,dout0,dout1,dout2,dout3,dout4,dout5,dout6,dout7);

input enb,a0,a1,a2;

output reg dout0,dout1,dout2,dout3,dout4,dout5,dout6,dout7;

always @ (enb,a0,a1,a2)

if (enb==0)

begin

casex({a0,a1,a2})

3'b111:begin

dout0 <= 0;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

dout7 <= 1;

end

3'b110:begin

dout0 <= 1;

dout1 <= 0;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

dout7 <= 1;

end

3'b101:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 0;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

dout7 <= 1;

end

3'b100:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 0;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

dout7 <= 1;

end

3'b011:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 0;

dout5 <= 1;

dout6 <= 1;

dout7 <= 1;

end

3'b010:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 0;

dout6 <= 1;

dout7 <= 1;

end

3'b001:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 0;

dout7 <= 1;

end

3'b000:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

dout7 <= 0;

end

default    :begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

dout7 <= 1;

end

endcase

end

else

begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

dout7 <= 1;

end

endmodule

（3）显示译码器

module a7448(enb,a0,a1,a2,a3,dout0,dout1,dout2,dout3,dout4,dout5,dout6);

input enb,a0,a1,a2,a3;

output reg dout0,dout1,dout2,dout3,dout4,dout5,dout6;

always @ (enb,a0,a1,a2,a3)

if (enb==0)

begin

casex({a0,a1,a2,a3})

4'b0000:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 0;

end

4'b0001:begin

dout0 <= 0;

dout1 <= 1;

dout2 <= 1;

dout3 <= 0;

dout4 <= 0;

dout5 <= 0;

dout6 <= 0;

end

4'b0010:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 0;

dout3 <= 1;

dout4 <= 1;

dout5 <= 0;

dout6 <= 1;

end

4'b0011:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 0;

dout5 <= 0;

dout6 <= 1;

end

4'b0100:begin

dout0 <= 0;

dout1 <= 1;

dout2 <= 1;

dout3 <= 0;

dout4 <= 0;

dout5 <= 1;

dout6 <= 1;

end

4'b0101:begin

dout0 <= 1;

dout1 <= 0;

dout2 <= 1;

dout3 <= 1;

dout4 <= 0;

dout5 <= 1;

dout6 <= 1;

end

4'b0110:begin

dout0 <= 0;

dout1 <= 0;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

end

4'b0111:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 0;

dout4 <= 0;

dout5 <= 0;

dout6 <= 0;

end

4'b1000:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

end

4'b1001:begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 0;

dout5 <= 1;

dout6 <= 1;

end

4'b1010:begin

dout0 <= 0;

dout1 <= 0;

dout2 <= 0;

dout3 <= 1;

dout4 <= 1;

dout5 <= 0;

dout6 <= 1;

end

4'b1011:begin

dout0 <= 0;

dout1 <= 0;

dout2 <= 1;

dout3 <= 1;

dout4 <= 0;

dout5 <= 0;

dout6 <= 1;

end

4'b1100:begin

dout0 <= 0;

dout1 <= 1;

dout2 <= 0;

dout3 <= 0;

dout4 <= 0;

dout5 <= 1;

dout6 <= 1;

end

4'b1101:begin

dout0 <= 1;

dout1 <= 0;

dout2 <= 0;

dout3 <= 1;

dout4 <= 0;

dout5 <= 1;

dout6 <= 1;

end

4'b1110:begin

dout0 <= 0;

dout1 <= 0;

dout2 <= 0;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

end

4'b1111:begin

dout0 <= 0;

dout1 <= 0;

dout2 <= 0;

dout3 <= 0;

dout4 <= 0;

dout5 <= 0;

dout6 <= 0;

end

default    :begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

end

endcase

end

else

begin

dout0 <= 1;

dout1 <= 1;

dout2 <= 1;

dout3 <= 1;

dout4 <= 1;

dout5 <= 1;

dout6 <= 1;

end

endmodule

（4）多路选择器

module mux41_a(y,a0,a1,a2,a3,s1,s0);

output y;

input a0,a1,a2,a3;

input s1,s0;

reg y;

always @(s1 or s0 or a0 or a1 or a2 or a3)

begin

case({s1,s0})

2'b00: y = a0;

2'b01: y = a1;

2'b10: y = a2;

2'b11: y = a3;

default:y =1'bx;

endcase

end

endmodule

- Quartus测试文件

（1）8线-3线优先编码器

// Copyright (C) 2018  Intel Corporation. All rights reserved.

// Your use of Intel Corporation's design tools, logic functions

// and other software and tools, and its AMPP partner logic

// functions, and any output files from any of the foregoing

// (including device programming or simulation files), and any

// associated documentation or information are expressly subject

// to the terms and conditions of the Intel Program License

// Subscription Agreement, the Intel Quartus Prime License Agreement,

// the Intel FPGA IP License Agreement, or other applicable license

// agreement, including, without limitation, that your use is for

// the sole purpose of programming logic devices manufactured by

// Intel and sold by Intel or its authorized distributors.  Please

// refer to the applicable agreement for further details.

// *****************************************************************************

// This file contains a Verilog test bench template that is freely editable to

// suit user's needs .Comments are provided in each section to help the user

// fill out necessary details.

// *****************************************************************************

// Generated on "03/04/2020 14:28:02"

// Verilog Test Bench template for design : logic2

//

// Simulation tool : ModelSim-Altera (Verilog)

//

`timescale 1 ns/ 1 ps

module logic2_vlg_tst();

// constants

// general purpose registers

//reg eachvec;

// test vector input registers

reg a0;

reg a1;

reg a2;

reg a3;

reg a4;

reg a5;

reg a6;

reg a7;

reg enb;

// wires

wire dout0;

wire dout1;

wire dout2;

// assign statements (if any)

logic2 i1 (

// port map - connection between master ports and signals/registers

.a0(a0),

.a1(a1),

.a2(a2),

.a3(a3),

.a4(a4),

.a5(a5),

.a6(a6),

.a7(a7),

.dout0(dout0),

.dout1(dout1),

.dout2(dout2),

.enb(enb)

);

initial

begin

// code that executes only once

// insert code here --> begin

enb = 0;

a0 = 0 ;

a1 = 0 ;

a2 = 0 ;

a3 = 0 ;

a4 = 0 ;

a5 = 0 ;

a6 = 0 ;

a7 = 0 ;

#50

a7=1;

a6=0;

#10

a6 = 1 ;

a5 = 1 ;

a4 = 0 ;

#30

a6 = 1 ;

a5 = 1 ;

a4 = 1 ;

a3 = 1 ;

a2 = 1 ;

a1 = 0 ;

# 40

enb = 1;

#20

enb = 0;

a0 = 1 ;

a1 = 1 ;

a2 = 1 ;

a3 = 1 ;

a4 = 1 ;

a5 = 1 ;

a6 = 1 ;

a7 = 1 ;

// --> end

$display("Running testbench");

end

endmodule

（2）3线-8线译码器

// Copyright (C) 2018  Intel Corporation. All rights reserved.

// Your use of Intel Corporation's design tools, logic functions

// and other software and tools, and its AMPP partner logic

// functions, and any output files from any of the foregoing

// (including device programming or simulation files), and any

// associated documentation or information are expressly subject

// to the terms and conditions of the Intel Program License

// Subscription Agreement, the Intel Quartus Prime License Agreement,

// the Intel FPGA IP License Agreement, or other applicable license

// agreement, including, without limitation, that your use is for

// the sole purpose of programming logic devices manufactured by

// Intel and sold by Intel or its authorized distributors.  Please

// refer to the applicable agreement for further details.

// *****************************************************************************

// This file contains a Verilog test bench template that is freely editable to

// suit user's needs .Comments are provided in each section to help the user

// fill out necessary details.

// *****************************************************************************

// Generated on "03/12/2020 11:17:19"

// Verilog Test Bench template for design : a74HC138

//

// Simulation tool : ModelSim-Altera (Verilog)

//

`timescale 1 ns/ 1 ps

module a74HC138_vlg_tst();

// constants

// general purpose registers

//reg eachvec;

// test vector input registers

reg a0;

reg a1;

reg a2;

reg enb;

// wires

wire dout0;

wire dout1;

wire dout2;

wire dout3;

wire dout4;

wire dout5;

wire dout6;

wire dout7;

// assign statements (if any)

a74HC138 i1 (

// port map - connection between master ports and signals/registers

.a0(a0),

.a1(a1),

.a2(a2),

.dout0(dout0),

.dout1(dout1),

.dout2(dout2),

.dout3(dout3),

.dout4(dout4),

.dout5(dout5),

.dout6(dout6),

.dout7(dout7),

.enb(enb)

);

initial

begin

// code that executes only once

// insert code here --> begin

enb = 0;

a0 = 0 ;

a1 = 0 ;

a2 = 0 ;

#50

a1=1;

a2=0;

#10

a2 = 1 ;

a0 = 1 ;

a1 = 0 ;

#30

a2 = 1 ;

a1 = 0 ;

# 40

enb = 1;

#20

enb = 0;

a0 = 1 ;

a1 = 1 ;

a2 = 1 ;

// --> end

$display("Running testbench");

end

endmodule

（3）显示译码器

// Copyright (C) 2018  Intel Corporation. All rights reserved.

// Your use of Intel Corporation's design tools, logic functions

// and other software and tools, and its AMPP partner logic

// functions, and any output files from any of the foregoing

// (including device programming or simulation files), and any

// associated documentation or information are expressly subject

// to the terms and conditions of the Intel Program License

// Subscription Agreement, the Intel Quartus Prime License Agreement,

// the Intel FPGA IP License Agreement, or other applicable license

// agreement, including, without limitation, that your use is for

// the sole purpose of programming logic devices manufactured by

// Intel and sold by Intel or its authorized distributors.  Please

// refer to the applicable agreement for further details.

// *****************************************************************************

// This file contains a Verilog test bench template that is freely editable to

// suit user's needs .Comments are provided in each section to help the user

// fill out necessary details.

// *****************************************************************************

// Generated on "03/13/2020 10:27:07"

// Verilog Test Bench template for design : a7448

//

// Simulation tool : ModelSim-Altera (Verilog)

//

`timescale 1 ns/ 1 ps

module a7448_vlg_tst();

// constants

// general purpose registers

//reg eachvec;

// test vector input registers

reg a0;

reg a1;

reg a2;

reg a3;

reg enb;

// wires

wire dout0;

wire dout1;

wire dout2;

wire dout3;

wire dout4;

wire dout5;

wire dout6;

// assign statements (if any)

a7448 i1 (

// port map - connection between master ports and signals/registers

.a0(a0),

.a1(a1),

.a2(a2),

.a3(a3),

.dout0(dout0),

.dout1(dout1),

.dout2(dout2),

.dout3(dout3),

.dout4(dout4),

.dout5(dout5),

.dout6(dout6),

.enb(enb)

);

initial

begin

// code that executes only once

// insert code here --> begin

enb = 0;

a0 = 0 ;

a1 = 0 ;

a2 = 0 ;

a3 = 0 ;

#100

a1=1;

a0=0;

#100

a0 = 1 ;

a1 = 1 ;

a2 = 0 ;

#100

a3 = 1 ;

a2 = 0 ;

a1 = 0 ;

#100

enb = 1;

#100

a3 = 0 ;

a2 = 1 ;

a1 = 0 ;

#100

enb = 0;

a0 = 1 ;

a1 = 1 ;

a2 = 1 ;

a3 = 1 ;

// --> end

$display("Running testbench");

end

endmodule

（4）多路选择器

// Copyright (C) 2018  Intel Corporation. All rights reserved.

// Your use of Intel Corporation's design tools, logic functions

// and other software and tools, and its AMPP partner logic

// functions, and any output files from any of the foregoing

// (including device programming or simulation files), and any

// associated documentation or information are expressly subject

// to the terms and conditions of the Intel Program License

// Subscription Agreement, the Intel Quartus Prime License Agreement,

// the Intel FPGA IP License Agreement, or other applicable license

// agreement, including, without limitation, that your use is for

// the sole purpose of programming logic devices manufactured by

// Intel and sold by Intel or its authorized distributors.  Please

// refer to the applicable agreement for further details.

// *****************************************************************************

// This file contains a Verilog test bench template that is freely editable to

// suit user's needs .Comments are provided in each section to help the user

// fill out necessary details.

// *****************************************************************************

// Generated on "03/06/2020 11:50:21"

// Verilog Test Bench template for design : mux41_a

//

// Simulation tool : ModelSim-Altera (Verilog)

//

`timescale 1 ns/ 1 ps

module mux41_a_vlg_tst();

// constants

// general purpose registers

//reg eachvec;

// test vector input registers

reg a0;

reg a1;

reg a2;

reg a3;

reg s0;

reg s1;

// wires

wire y;

// assign statements (if any)

mux41_a i1 (

// port map - connection between master ports and signals/registers

.a0(a0),

.a1(a1),

.a2(a2),

.a3(a3),

.s0(s0),

.s1(s1),

.y(y)

);

initial

begin

a0=2'b00;

a1=2'b01;

a2=2'b10;

a3=2'b11;

s1=1'b0;s0=1'b0;

#100

s1=1'b0;s0=1'b1;

#100

s1=1'b1;s0=1'b0;

#100

s1=1'b1;s0=1'b1;

#100;

end

endmodule

- Quartus仿真效果图

（1）8线-3线优先编码器

（2）3线-8线译码器

- （3）显示译码器

（4）多路选择器